Annex -5

GUIDELINES ‘A’

for preparing Municipality Transport Master Plan (MTMP)

Damak Municipality,Damak Jhapa.

- Prepare visionary city development plan with the consultation of stakeholders taking input from expert, if required.

- GPS shall be used for collection of relevant data on road name, code, length, surface type, and width for road inventory followed by digitizing in GIS platform.

- Nodal point such as municipality building, ward building, bus park, historical place, play ground, health post, river crossing shall be taken as base for digital naming of the road, junctions with strategic and DRCN. The digital naming is given in road inventory format.

- Municipality road are classified as

- Main Collector Road------- class A; RoW -14m

- Other Collector Road --------class B; RoW – 10m

- Main Tole Road     ------------class C; RoW - 6m

- Other Road---------------------class  D; RoW - 4 m

Other national highways, feeder road and district core network passing through municipality shall have right of way to their respective standards. Building line/set back shall be maintained 6m for roads having RoW equal and /or more than 20m and 2m for other roads.

- During preparation of MTMP, the investment from total available resources under road sector for different classes of the road can be distributed as

- Main Collector Road------- class A; ------------------40%

- Other Collector Road --------class B; -----------------30%

- Main Tole Road     ------------class C; ----------------20%

- Other Road---------------------class  D; ----------------10%

- Do not take urban road having road corridor width less than 3.75m.

- Do not consider dead end road having length more than 150m.

- Take road surface as asphalt concrete and RCC for metalled and Gravel and Earthen for others from the priority list of municipality and also based on its functionality and available resources.

- Formulate municipality road coordination committee (MRCC) as follows:

Mayor 							Chairperson MRCC

Executive Officer of municipality				Member

Two elected or nominated Municipality members 		Member

One representative from different political parties	          Member

Chiefs of Lines agencies within the municipality		         Member

Representative from Women and ethnic minority groups	         Member

DTO representative					         Member

Planning section chief of municipality			         Member

MTS Chief						        Member Secretary

- Consult MoFALD/IDD/DoLIDAR during preparation if any difficulty.

- Steps:

- Hire the consultant

- Form the MRCC

- Prepare/Finalize visionary city development plan

- Finalize IMDPP

- Finalize MIM

- Finalize MTPP & MTMP

12.  The standard cost for different intervention can be taken as:

Table 1	Standard costs for different interventions

| Activity | Unit | Unit cost (NPR) |
| --- | --- | --- |
| Emergency maintenance | km | 30,000 |
| Routine maintenance | km | 20,000 |
| Recurrent maintenance (blacktop) | km | 500,000 |
| Recurrent maintenance (gravel) | km | 400,000 |
| Recurrent maintenance (earthen) | km | 250,000 |
| Periodic maintenance (blacktop) | km | 200,000 |
| Periodic maintenance (gravel) | km | 250,000 |
| Rehabilitation | km | 800,000 |
| Widening | m | 25,000 |
| Gravelling | km | 2,200,000 |
| Blacktopping | km | 5,700,000 |
| Bridge construction | m | 600,000 |
| Slab culvert construction | m | 150,000 |
| CC Causeway construction | m | 100,000 |
| Stone Causeway construction | m | 10,000 |
| Pipe culvert placement | unit | 10,000 |
| Masonry wall construction | m3 | 10,000 |
| Gabion wall construction | m3 | 2,500 |
| Lined drain construction | m | 1,000 |
| Track opening | km | 4,000,000 |
| Gravelling | km | 2,200,000 |
| Bridge construction | m | 600,000 |

Abbreviations :

MRCC : Municipality Road Co-ordination committee.

IMDPP :

MIM : Municipality Inventory Map

MTPP :

MTMP : Municipality Transportation Master Plan

IMDPM : Indicative Municipality Development Potential Map

Reference taken from Urban Development Standard Planning, 2014/MoUDB

Referance taken from DoLIDAR Approach Manual